Routine Specifications

- Note: Please remember to use a mask when changing specific bits in a register!

- __main

  - Configure all inputs and outputs (reference each lab we did for configuring push-pull/open-drain and things like that), initialize register values (0x0 for R4-R7, 0x00000900 for R8, and 0x00000008 for R9), then write a “begin” label after all the configurations & initializations are done

  - Write a “1” (on) to the “system idle” bit in R8

  - Branch with link to update_LED_states

  - Display “Press ‘#’ to Start” message on Tera Term.

  - Scan for a “#” input on the keypad

    - When “#” has been pressed:

      - Write a “0” (off) to the “system idle” bit in R8

      - Branch with link to update_LED_states

      - Prompt the user to press “0” if exiting the garage or “1” if entering the garage

    - If “1” is selected:

      - Prompt the user to press “*” for manual operation, “#” for automatic operation, or “9” for priority parking

      - If “*” is pressed, branch to manual_sequence

      - If instead “#” is pressed, branch to automatic_sequence

      - If instead “9” is pressed, branch to priority_sequence

    - If instead “0” is selected:

      - Branch to exit_sequence

  - Branch back to the “begin” label

- exit_sequence

  - Prompt the user to input 1, 2, 3, or 4 (reserved level) from the keypad to move the platform to that level

  - Scan for input 1, 2, 3, or 4 on the keypad, and store the selection in a register R0-R3

  - Check whether the selected parking level is empty by referring to the parking spots tracking in R8

    - If the level is empty, display “level is empty” and branch back to the “begin” label in __main

    - Otherwise, continue

  - Update R5 with the motor position of the selected level (4096, 8192, 12288, or 16384, in steps) by referring to register R0-R3

  - Branch with link to move_platform

  - Prompt the user to input “#” when the car has entered the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is on the platform

  - Branch with link to increment_open_spots

  - Update R8 with the new number of cars at the selected level

  - Update R5 with the motor position of the ground level (0, in units of steps)

  - Branch with link to move_platform

  - Branch with link to open_gate

  - Prompt the user to input “#” when they have exited the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is off the platform

  - Branch with link to close_gate

  - Branch back to the “begin” label in __main

- manual_sequence

  - Prompt the user to input 1, 2, or 3 from the keypad to select the platform destination level

  - Scan for input 1, 2, or 3 on the keypad, and store the selection in a register R0-R3

  - Check whether the selected parking level is full by referring to the parking spots tracking in R8

    - If the level is full, display “level is full” and branch back to the “begin” label in __main

    - Otherwise, continue

  - Update R5 with motor position of the ground level (0, in units of steps)

  - Branch with link to move_platform

  - Branch with link to open_gate

  - Prompt the user to input “#” when the car has entered the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is on the platform

  - Branch with link to close_gate

  - Update R5 with the motor position of the selected level (4096, 8192, 12288, or 16384, in steps) by referring to register R0-R3

  - Branch with link to move_platform

  - Prompt the user to input “#” when they have exited the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is off the platform

  - Branch with link to decrement_open_spots

  - Update R8 with the new number of cars at the selected level

  - Branch back to the “begin” label in __main

- automatic_sequence

  - If R8 shows that the garage is full (levels 1-3 are full, reserved level may be open), display message to Tera Term that “the unreserved garage is full” and branch back to the “begin” label in __main

  - Otherwise, if the garage is not full:

    - Check R8 to see whether level 1 is full  if level 1 is full, check whether level 2 is full  if level 2 is full, then level 3 must be open

    - Store the lowest open level (as determined in the above check) in a register R0-R3

  - Update R5 with motor position of the ground level (0, in units of steps)

  - Branch with link to move_platform

  - Branch with link to open_gate

  - Prompt the user to input “#” when the car has entered the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is on the platform

  - Branch with link to close_gate

  - Update R5 with the motor position of the selected level (4096, 8192, 12288, or 16384, in steps) by referring to register R0-R3

  - Branch with link to move_platform

  - Prompt the user to input “#” when they have exited the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is off the platform

  - Branch with link to decrement_open_spots

  - Update R8 with the new number of cars at the selected level

  - Branch back to the “begin” label in __main

- priority_sequence

  - Prompt the user to input a code to access the reserved level (the code will be “6767”)

  - Scan for a “6” and a “7” in sequence twice

    - Use a register R0-R3 to store 1 or 0 depending on whether the user hasn’t/has input a wrong number

    - Reject a wrong input and display rejection to Tera Term, then branch back to the “begin” label in __main

    - For the correct sequence, display acceptance to Tera Term, then continue

  - Check whether R8 shows if the reserved level is full

    - If the reserved level is full (has 2/2 spots filled), display “reserved level is full” message to Tera Term and branch back to the “begin” label in __main

    - Otherwise, continue

  - Update R5 with the motor position of the ground level (0, in units of steps)

  - Branch with link to move_platform

  - Branch with link to open_gate

  - Prompt the user to input “#” when they have entered the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is on the platform

  - Branch with link to close_gate

  - Update R5 with the motor position of the reserved (4th) level (16384, in steps)

  - Branch with link to move_platform

  - Prompt the user to input “#” when they have exited the platform, and keep checking whether they enter a “#” until the key has been pressed

    - Update R8 to indicate that a car is off the platform

  - Branch with link to decrement_open_spots

  - Update R8 with the new number of cars at the reserved level

  - Branch back to the “begin” label in __main

- open_gate

  - Push the LR onto the stack

  - Update R7 with a 135-degree motor position (1536 steps)

  - Write to Tera Term “opening gate”

  - Until the current gate position matches the desired gate destination position:

    - Continually update R6 to store the current position (in steps) of the gate motor with each loop of the stepping sequence

    - Load R6 onto a register R0-R3 to keep track of the current motor position (in steps) and compare with the desired destination position

    - Output the stepping sequence to the gate stepper motor pins

  - Write a “1” (on) to the “gate status” bit in R8

  - Branch with link to update_LED_states

  - Pop the LR from the stack

  - Branch indirect (BX)

- close_gate

  - Push the LR onto the stack

  - Update R7 with a 0-degree motor position (0 steps)

  - Write to Tera Term “closing gate”

  - Until the current gate position matches the desired gate destination position:

    - Continually update R6 to store the current position (in steps) of the gate motor with each loop of the stepping sequence

    - Load R6 onto a register R0-R3 to keep track of the current motor position (in steps) and compare with the desired destination position

    - Output the stepping sequence to the gate stepper motor pins

  - Write a “0” (off) to the “gate status” bit in R8

  - Branch with link to update_LED_states

  - Pop the LR from the stack

  - Branch indirect (BX)

- move_platform

  - Push the LR onto the stack

  - Write a “0” (off) to the “platform status” bit in R8

  - Branch with link to update_LED_states

  - Write to Tera Term “moving platform…”

  - Until the current platform position in R4 matches the desired platform destination position in R5:

    - Continually update R4 to store the current position (in steps) of the platform motor with each loop of the stepping sequence

    - Load R4 onto a register R0-R3 to keep track of the current motor position (in steps) and compare with the desired destination position

    - Output the stepping sequence to the platform stepper motor pins, using acceleration and deceleration (from increasing and decreasing step delay) during the sequence

    - If the current platform motor position in R4 is either [0, 4096, 8192, 12288, or 16384], branch with link to display_level

  - Write a “1” (on) to the “platform status” bit in R8

  - Branch with link to update_LED_states

  - Pop the LR from the stack

  - Branch indirect (BX)

- display_level

  - Translate the current platform motor position from R4 (0, 4096, 8192, 12288, or 16384, in steps) into its corresponding level (0-4), and store that level in the platform level bits in R9

  - Map the platform level bits from R9 onto the BCD platform level display pins

  - Write to Tera Term the new “Platform level: x”

  - Branch indirect (BX)

- increment_open_spots

  - Translate the parking spots tracking bits from R8 into the number of spots open in the garage

  - Subtract 1 from the number of open spots in the garage

  - Update R9 with the number of spots open in the garage

  - Write to Tera Term the new “parking spots available: x”

  - Map the corresponding bits from R9 onto the BCD open spots display pins

  - Branch indirect (BX)

- decrement_open_spots

  - Translate the parking spots tracking bits from R8 into the number of spots open in the garage

  - Add 1 to the number of open spots in the garage

  - Update R9 with the number of spots open in the garage

  - Write to Tera Term the new “parking spots available: x”

  - Map the corresponding bits from R9 onto the BCD open spots display pins

  - Branch indirect (BX)

- update_LED_states

  - Mask the LED state bits from R8 onto their corresponding output pins

  - Branch indirect (BX)

- stop_mechanism

  - When this interrupt is triggered, it will first prompt the user on Tera Term to hit the “#” key to reset the mechanism or hit “*” to go back to what the program was doing

    - If the user instead presses the “*” key, end the ISR

    - If the user presses the “#” key, branch with link to reset_mechanism, and rewrite the return value for LR in the stack to the “begin” label in the __main function

  - End the ISR

- reset_mechanism

  - Push the LR onto the stack

  - Write to Tera Term that the system is resetting

  - Blink the emergency reset LED off and on 3 times by doing the following for 3 iterations:

    - Write a “0,” then delay, and write a “1” to the “emergency reset” bit in R8

    - After writing a “0” or a “1,” branch with link to update_LED_states

  - Update R5 with the motor position of the ground level (0, in units of steps)

  - Branch with link to move_platform

  - Check whether there is a car on the platform from R8:

    - If there is a car on the platform, branch with link to open_gate, and prompt the car to exit the platform and to input “#” upon exiting, and keep checking whether they enter a “#” until the key has been pressed

    - Branch with link to close_gate

  - Write a “0” (off) to the “emergency reset” bit in R8

  - Write to Tera Term that the system has reset

  - Pop the LR from the stack

  - Branch indirect (BX)

Registers

- R4: Current platform motor position [units of steps from 0 (4096 steps = 1 rotation)]

- R5: Desired platform motor destination [units of steps from 0 (4096 steps = 1 rotation)]

- R6: Current gate motor position [units of steps from 0 (4096 steps = 1 rotation)]

- R7: Desired gate motor position [units of steps from 0 (4096 steps = 1 rotation)]

- R8: LED states, parking spots tracking, & platform status tracking

  - Bits 31-24 store the number of cars parked at level “reserved” and levels 3, 2, 1

    - I.e., if 31-30 is “00,” there are 0/2 cars on the reserved level; if 29-28 is “10,” there are 2/2 cars on level 3 (we’ll do 2 cars on each level for simplicity)

  - Bits 2, 8, 11, & 12 store the “on” (1) and “off” (0) states of the emergency reset, system idle, platform status, and gate status LEDs, respectively

    - Emergency Reset: 1 = System stopped, 0 = System uninterrupted, 3 Blinks = System resetting

    - System Idle: 1 = Idle, 0 = Operating

    - Platform Status: 1 = Ready/not moving, 0 = Moving

    - Gate Status: 1 = Open, 0 = Closed

  - Bit 0 stores a “0” if there is no car on the platform and a “1” if there is a car on the platform

- R9: BCD display values

  - Bits 0-3 store the number of spots open in the garage (will be 0-8 spots)

  - Bits 4-6 store the current level of the platform (“100” (4) will be considered the reserved level, and “000” (0) will be considered the ground level)

Pins

Gate Motor:

- 4 Outputs: PB4 – PB7 (Magnet A  Magnet D)

Platform Motor:

- 4 Outputs: PB0 – PB3 (Magnet A  Magnet D)

Keypad:

- 3 Inputs: PC0 – PC2 (Col 1  Col  3)

- 4 Outputs: PA4 – PA7 (Row 1  Row 4)

Level Display:

- 4 Outputs: PC3 – PC6 (LSB  MSB)

Open Spots Display:

- 4 Outputs: PC7 – PC10 (LSB  MSB)

Gate Status LED:

- 1 Output: PC11

Platform Status LED:

- 1 Output: PC12

Emergency Reset LED:

- 1 Output: PD2

System Operating LED:

- 1 Output: PA8

Stop & Reset Pushbutton:

- 1 Input: PA0

Priority Request Pushbutton:

- 1 Input: PA1

Total Pins: 5 Inputs, 24 Outputs

Outline with Diagrams

Project Specifications (from Canvas)

- The prototype must include the Nucleo-STM32L476RG microcontroller, stepper motor(s) to mimic the motion of the elevator platform and parking gate, push button(s) and/or keypad(s) for driver and operator commands, seven-segment display(s) to show the current parking level, and LEDs to indicate system status.

- The main program of the microcontroller should be primarily written in assembly language.

- One stepper motor should simulate the elevator platform that moves vehicles between parking levels.

- A second motor should simulate the garage entry gate, opening and closing when vehicles enter or exit.

- The system must support at least three parking levels (Level 1, Level 2, Level 3).

- The system should automatically assign vehicles to parking levels and move the platform accordingly.

- The operator should be able to manually override the automated system and move the platform to a specific level using push buttons or keypad input.

- The seven-segment display should indicate the current parking level or system status.

- LEDs should indicate states such as platform moving, gate open, or system idle.

- System status and events should be communicated in real time through Tera Term.

- The hardware layout and wiring should be neat and organized.

- Software should be properly annotated.

Bonus:

- Implement acceleration/deceleration of the platform using step delay or PWM.

- Display the number of available parking spots.

- Implement a priority parking request (e.g., reserved level).

- Add an emergency button for stopping and resetting the mechanism.